PORTFOLIO PROJECT REPORT

RadioML Modulation
Classifier

An interpretable RF signal intelligence pipeline using classical machine learning

| Project | Dataset | Approach |
| --- | --- | --- |
| RadioML modulation recognition | RadioML 2016.10a | Handcrafted features + classical ML |

Prepared by Chandranshu Verma

August 2026

# Executive summary

This project implements an end-to-end modulation-recognition workflow for short complex baseband captures. It begins with raw in-phase and quadrature samples, converts each capture into a compact set of interpretable RF measurements, trains two classical classifiers, evaluates performance across signal-to-noise ratio, adds unsupervised anomaly detection, and exposes the pipeline through a small Streamlit application.

The goal is explainability rather than state-of-the-art accuracy. Logistic Regression provides a linear baseline, while Random Forest captures nonlinear relationships among amplitude, phase, and spectral features. On the fixed held-out test set, Logistic Regression achieved 44.45% accuracy and Random Forest achieved 50.44%. Random Forest performance rose from approximately chance level at -20 dB to 79.1% at 18 dB, demonstrating that aggregate accuracy alone is not an adequate RF performance measure.

| Measure | Verified result | Interpretation |
| --- | --- | --- |
| Dataset size | 220,000 signals | Balanced across modulation and SNR |
| Signal shape | 2 × 128 | I and Q channels, 128 samples each |
| Modulation classes | 11 | Analog and digital modulation families |
| SNR range | -20 to 18 dB | 20 levels in 2 dB steps |
| Random Forest accuracy | 50.44% | All SNR levels combined |
| Random Forest at 18 dB | 79.14% | Clear-signal operating point |

## Project objectives

# 1. Dataset and signal representation

RadioML 2016.10a is stored as a dictionary keyed by (modulation, SNR). Each key maps to 1,000 examples with shape (1000, 2, 128). A single example therefore contains 128 I samples and 128 Q samples. The inspected file contains 220 dictionary blocks and 220,000 signals in total.

## I/Q samples

A complex baseband sample is represented as x[n] = I[n] + jQ[n]. The I component lies on the horizontal axis and the Q component lies on an orthogonal axis. Together they retain both signal magnitude and phase. Magnitude is calculated as |x[n]| = sqrt(I[n]^2 + Q[n]^2), while phase is calculated with atan2(Q[n], I[n]).

## Modulation classes

| Family | Classes in the dataset |
| --- | --- |
| Phase/amplitude digital | BPSK, QPSK, 8PSK, QAM16, QAM64, PAM4 |
| Frequency digital | CPFSK, GFSK |
| Analog | AM-DSB, AM-SSB, WBFM |

## Signal-to-noise ratio

SNR compares signal power with noise power: SNR(dB) = 10 log10(Psignal / Pnoise). At negative SNR, noise power exceeds signal power. In that regime, constellation structure, phase transitions, amplitude patterns, and spectral shape become difficult to distinguish. This is why modulation-recognition performance must be reported as a function of SNR.

Figure 1. QPSK example at 18 dB: time-domain channels, amplitude, I/Q scatter, and FFT.

# 2. Feature engineering

Each raw capture contains 256 scalar values. Instead of passing these values directly to a model, the project compresses each capture into 15 measurements that can be explained in RF and statistical terms. This loses fine time-order information, but produces a structured table suitable for classical machine learning.

| Feature group | Measurements | Purpose |
| --- | --- | --- |
| Amplitude | mean, standard deviation, RMS, maximum, variance | Signal strength and variation |
| Power | mean power | Average I² + Q² |
| Phase | phase standard deviation | Angular spread |
| I/Q | I std, Q std, I/Q correlation | Variation and channel relationship |
| Spectrum | centroid, bandwidth, entropy, FFT peak | Frequency location and energy distribution |
| Distribution | amplitude kurtosis | Peakedness and heavy tails |

## Spectral entropy

The FFT power spectrum is normalized into a probability distribution across frequency bins. Entropy is then divided by log2(N), producing a value between zero and one. Low entropy means energy is concentrated in relatively few bins; high entropy means energy is broadly distributed and more noise-like.

## Feature-table checks

# 3. Classification methodology

The data was divided into 176,000 training signals and 44,000 test signals. Stratification used the combined modulation and SNR label, preserving every operating condition in both partitions. SNR was excluded from the feature matrix and retained only for analysis.

## Models

| Model | Role | Configuration |
| --- | --- | --- |
| Logistic Regression | Linear baseline | StandardScaler + LogisticRegression; 1,000 max iterations |
| Random Forest | Main classifier | 100 trees; maximum depth 18; minimum leaf size 2 |

Standardization is performed inside the Logistic Regression pipeline, so scaling parameters are learned from training data only. Random Forest does not require feature scaling because its trees use threshold splits. Limiting tree depth reduced the serialized classifier from roughly 1.4 GB to 178 MB while slightly improving measured accuracy.

## Evaluation measures

## Overall test results

| Model | Accuracy | Gain over chance |
| --- | --- | --- |
| Logistic Regression | 44.45% | +35.36 percentage points |
| Random Forest | 50.44% | +41.35 percentage points |

With 11 balanced classes, random guessing corresponds to approximately 9.09%. The nonlinear forest improves on the linear baseline, indicating that useful relationships among the handcrafted features are not purely linear.

# 4. Results and interpretation

Figure 2. Test accuracy by SNR for both classifiers.

| SNR | Random Forest accuracy | Operating interpretation |
| --- | --- | --- |
| -20 dB | 9.1% | Near chance |
| -10 dB | 17.4% | Near chance |
| -6 dB | 40.4% | Transition region |
| 0 dB | 67.3% | Transition region |
| 6 dB | 77.5% | High-SNR plateau |
| 12 dB | 79.5% | High-SNR plateau |
| 18 dB | 79.1% | High-SNR plateau |

The curve has three clear regions. Below approximately -14 dB, the extracted features contain little usable class information. Between -10 dB and 2 dB, accuracy rises rapidly as modulation structure emerges from the noise. Above approximately 6 dB, the forest plateaus near 78–80%. This plateau suggests that the remaining errors are caused primarily by representational limits rather than insufficient SNR.

## Confusion analysis

Figure 3. Random Forest confusion matrix, normalized by true class.

## Feature importance

Figure 4. Random Forest mean-decrease-in-impurity feature importance.

Spectral entropy and spectral bandwidth are the leading features, followed by Q variation, FFT peak magnitude, and I/Q correlation. Mean amplitude contributes very little, matching the earlier observation that RadioML normalizes average magnitude. Importance values are diagnostic rather than causal: correlated measurements such as RMS amplitude and mean power can divide importance between themselves.

# 5. Anomaly detection

Isolation Forest was trained on a reproducible sample of 50,000 ordinary training examples using the same 15 features. It was not given modulation labels or anomaly labels. The detector returns a decision score: positive values are treated as normal and negative values as anomalous. A contamination setting of 2% establishes the operating threshold.

| Test group | Mean score | Flagged anomalous | Result |
| --- | --- | --- | --- |
| Normal high-SNR signals | +0.087 | 3.3% | Low false-positive rate |
| Strong added noise | -0.094 | 100.0% | Detected |
| Threefold amplitude | -0.105 | 100.0% | Detected |
| Frequency offset | +0.076 | 2.7% | Mostly missed |

Figure 5. Isolation Forest score distributions. The dashed line marks the decision threshold.

## Interpretation

Added noise and amplitude scaling move several features outside the training distribution, so they are isolated reliably. A pure frequency shift mainly changes spectral centroid while preserving amplitude, power, entropy, and bandwidth. The detector therefore accepts most shifted signals. In a practical spectrum-monitoring system, this unsupervised detector should be paired with explicit allowed-channel or carrier-frequency rules.

# 6. Inference and application

The reusable predict_signal(iq_signal, snr) function validates a (2, 128) input, extracts features, invokes the Random Forest classifier, obtains its maximum class probability, calculates an Isolation Forest decision score, and returns one dictionary. SNR is returned as context but is not provided to either model.

| Output field | Meaning |
| --- | --- |
| predicted_modulation | Winning Random Forest class |
| confidence | Largest predicted class probability; not formally calibrated |
| anomaly_score | Isolation Forest decision score |
| is_anomaly | True when the anomaly prediction is -1 |
| snr_db | Measurement context returned to the caller |

## Example behavior

| Input | Prediction | Confidence | Anomaly result |
| --- | --- | --- | --- |
| QPSK at 18 dB | QPSK | 59.7% | Normal, score +0.137 |
| QAM16 at -20 dB | AM-SSB | 26.8% | Normal, but uncertain class |
| Noisy QPSK variant | QAM64 | 16.4% | Anomaly, score -0.086 |

The noisy example illustrates why classification and anomaly detection are complementary. The classifier must select one of its 11 known classes, even when none is a good match. The anomaly detector supplies an additional warning that the feature vector lies outside the normal training distribution.

## Streamlit dashboard

The local dashboard allows selection of modulation, SNR, and example index. It displays I/Q waveforms, amplitude, I/Q scatter, FFT magnitude, predicted class, confidence, anomaly score, the SNR accuracy curve, and the Random Forest confusion matrix. It calls the same inference function used by command-line tests, avoiding duplicate model logic.

# 7. Limitations and next steps

## Reasonable extensions

# 8. Interview summary

I built a classical RF machine-learning pipeline on RadioML 2016.10a. Each input is a short complex baseband capture with 128 I/Q samples. I converted it into 15 interpretable amplitude, phase, correlation, and spectral features, then compared Logistic Regression with Random Forest using a split stratified by modulation and SNR. Random Forest reached 50.44% overall accuracy, but the key result was the SNR curve: performance was near chance at very low SNR and approximately 79% at 18 dB. Spectral entropy and bandwidth were the most useful features. I also added Isolation Forest to flag unusual feature combinations, and I documented that it detected strong noise and amplitude anomalies but usually missed pure frequency shifts. Finally, I wrapped classification and anomaly scoring in one inference function and a small Streamlit interface.

# 9. Reproduction

From the repository root, create the environment, set RADIOML_DATASET to the trusted pickle location, and run the following commands in order:

python scripts/explore.py "$RADIOML_DATASET"

python scripts/build_features.py "$RADIOML_DATASET"

python src/train.py

MPLBACKEND=Agg python src/evaluate.py

MPLBACKEND=Agg python src/anomaly.py "$RADIOML_DATASET"

streamlit run app.py

## Artifact map

| Location | Contents |
| --- | --- |
| data/ | Feature table and local dataset link |
| models/ | Classifier, split IDs, and anomaly detector |
| results/ | Metrics, reports, and figures |
| scripts/ | Exploration and feature-building commands |
| src/ | Feature extraction, training, evaluation, anomaly, and inference code |